Geelong Kart Club Class Structure                v4.5       16/10/2024

- NOVICE  -  Ages 6 to 9

Must go to Rookies upon 10th birthday or at end of year.

Number Plate:  White backing plate with red numbers.

Engines

Comer SW 80

Yamaha 100 J with Cadet 9 Restrictor (13mm)

Vortex Mini Rok with Cadet 9 Restrictor (16 mm)

Clutches on Yamaha mandatory for racing and practise.   Engines to conform to OEM.

Tyres

Dry – Dunlop SL3 Cadet

Wets – Dunlop KT3-W7 Cadet

Weights

Vortex Mini Rok - 99 kgs

Yamaha J - 97 kgs

Comer SW80 – 90 kgs

- ROOKIES - Ages 9 to 12

Must go up upon 12th birthday or at end of year.

Number Plate: White backing plate with red numbers.

Engines

Vortex Mini Rok

Yamaha J with Restrictor  (16mm)

Clutches are mandatory on Yamaha for racing and practise.  Engines to conform to OEM.

Tyres

Dry - Dunlop SL3 Cadet

Wets - Dunlop KT3-W7 Cadet

Weights

Vortex Mini Rok - 110 kgs

Yamaha 100 J - 102 kgs

- JUNIOR RESTRICTED  - Ages 12 to 15

Must go up upon 16th birthday or at the end of year.

Number Plate:  White backing plate with Black numbers.

Engines

Iame 100 Reed jet with restrictor – (22mm)

Yamaha 100 J

Yamaha 100 S

ARC Spec 100A

ARC Spec 100A / Yamaha 100 S Hybrid

Clutches are mandatory for racing and practise at all times.

Engines to conform to OEM.

Tyres

Dry -  Dunlop DFM

Wets – Dunlop KT14W13

Weights

Light - 132 kgs for Iame Reed jet

Heavy -  149 kgs for Iame Reed jet

Light - 110 kgs for Yamaha J

Heavy -  130 kgs for Yamaha J

Light - 132 kgs for Yamaha S

Heavy -  149 kgs for Yamaha S

Light - 132 kgs for ARC Spec 100 A or hybrid with ARC/Yam S

Heavy -  149 kgs for ARC Spec 100 A or hybrid with ARC/Yam S

- Open Performance

Age 15 and Over.

Number Plate:  Yellow backing plate with Black numbers.

Engines

Any Karting Homologated Engine including twin variants.

KZ2 not permitted unless authorised by GKC at specific event due to combining class etc.

Tyres

Dry – LeCont SVC/LMP or MG Yellow SM or MG Yellow SM2

Wets - Any homologated Karting Wet tyre 5 inch only

Weights:

Minimum 150 kgs all single engine non gearbox – up to 101 cc

Minimum 160 kgs all single engine non gearbox - up to 125cc

Minimum 165 kgs all open 4 stroke up to 252cc

Minimum 170 kgs all single engine non gearbox  - up to 175cc (165 kgs if no front brakes)

Minimum 170 kgs all DD2

Minimum 175 kgs twins – Yamaha S, ARC air and water cooled, KA100 - Air cooled reed Formula 100 engines.

Minimum 185 kgs twins.  Any water cooled reed or rotary and X30 / Rotax Max twins.

Open class engines may be bored to larger capacity if CC limit not listed above.

- Tag 125 Restricted

Ages 15 and up.

Number Plate:  Yellow backing plate with Black numbers.

Engines

Engines to conform to OEM - Listed below with maximum restrictor size allowed.

IAME X30 23.40 mm for Type 1 (older exhaust) 22.7 mm for Type 2 (New exhaust)

PRD Fireball 125 27.00 mm

PRD Galaxy 125 25.50 mm

Parilla Leopard 125-Black Box 23.00 mm

Parilla Leopard 125-Green Box 23.80 mm

Rotax 125 Max 24.50 mm – Plus 2kgs for Evo

Rotax 125 Junior Max 24.50 mm

Max Number of engines is 2.  Different engines types can be used.

Tyres

Dry - Dunlop DFM

Wets - Dunlop KT14W13

Weights

Light  - 160 kgs

Heavy - 180 kgs

Rotax Evo with 24.50 mm Restrictor – Plus 2kgs

- Tag 125 Open

Ages 15 and up.

Number Plate:  Yellow backing plate with Black numbers.

Engines

Engines to conform to OEM

IAME X30

PRD Fireball 125

PRD Galaxy 125

Parilla Leopard 125

Rotax Max 125 (non EVO and EVO)

Tyres

Dry - Dunlop DFM

Wets - Dunlop KT14W13

Weights

Light  - 160 kgs

Heavy -  180 kgs

Max Number of engines is 2.  Different engines types can be used.

- KZ2

Ages 15 and up.

Number Plate:  Yellow backing plate with Black numbers.

Engines

Engines to conform to OEM

125cc water cooled, single cylinder, reed valve which has been homologated by CIK-FIA currently or in the past.

Carburettor – Dell Orto VHSH 30

Max number of engines permitted at a single meeting is three.

OEM applies to all ancillaries including fuel pump and electrics.

Tyres

Dry – LeCont SVC/LMP or MG Yellow SM or MG Yellow SM2

Wets – LeCont SV1 or any homologated Karting Wet tyre 5 inch only

Weights  -    177 kgs Minimum

- Clubman  Senior  ( Light and Heavy )

Ages 15 and up.

Number Plate:  Yellow backing plate with Black numbers.

Engines

Yamaha 100 S

ARC Spec 100A

ARC Spec 100W

ARC Spec 100 A/W / Yamaha Hybrid

Iame Reed jet 100

Engines to conform to OEM.  Clutch not mandatory on Yamaha

Tyres

Dry - Dunlop DFM  or for Non KA100 Engines Dunlop DFM or MG Red AZ.

Wets - Dunlop KT14W13 or MG White.

Weights

Light 154 kgs Iame / Yamaha 137 kgs / ARC Air cooled or hybrid air cooled 137 kgs / ARC 100 Water cooled 142 kgs

Heavy 174 kgs Iame / Yamaha 157 kgs / ARC Air cooled or hybrid air cooled 157 kgs / ARC 100 Water cooled 162kgs

- Yamaha S

Ages 15 and up.

Number Plate:  Yellow backing plate with Black numbers.

Engines

Yamaha 100 S

Engines to conform to OEM.  Clutch not mandatory on Yamaha

Tyres

Dry - Dunlop DFM or MG Red AZ

Wets - Dunlop KT14W13 or MG White.

Weights

Light 140 kgs

Heavy 160kgs

- Clubman  Super Heavy

Ages 15 and up.

Number Plate:  Yellow backing plate with Black numbers.

Engines

Yamaha 100 S

ARC Spec 100A

ARC Spec 100W

Engines to conform to OEM.  Clutch not mandatory.

Tyres

Dry - Dunlop DFM or MG Red AZ

Wets - Dunlop KT14W13 or MG White.

Weights

Yamaha S and ARC Aircooled – 180kgs

ARC Watercooled – 185kgs

- Four Stroke Senior

15 and up or year of 15th birthday.

Number Plate:  Yellow backing plate with Black numbers.

Engines

Torini Clubmaxx,

Torini Supermaxx

Tillotson 225 RS

All engines must have manufacturer engine seals for all GKC race events.

All clutches must be OEM.

Tyres

Dry  - MG Red SH  or Maxxis Supersport 4SS

Wets -  Dunlop KT14W13 or MG White.

Weights

Torini Clubmaxx 150kg,

Torini Supermax and Tillotson RS225 175kg

Tyre Rules / Dimensions For All Classes -  All  tyres must be :

1) Front Tyre size must be a. 10 x 4.50-5 2) - Dry

Rear Tyre size must be a. 11 x 7.10-5 - Dry

2)  Wet tyres must used 5 inch rims only and can be maximum dry tyre size or smaller.

3)  No tyre treatment of any type of design will be allowed in any class.  Penalties will apply.

4)  Wet and Dry tyres can be used at any time and mixed and matched at the choice of competitor.

5)  Only one set of Dry tyres and one set of Wet tyres can be used per race meeting unless specified differently in supp regs.

6) Tyres may be marked at any time at the discretion of the club technical officers or tyre

scanning can also be used at any time.

7) For racing and practise all karts must only have one complete matching set of tyres of the same brand and tyre type on the kart at any time.  Any club official can check this at any time including during non race days.

8) Age and weight amendments for individual drivers will be based on merit of application.  Final decision by AASA GKC representative.

9) As bumper bars can be plastic latest type or traditional steel bumpers there is no requirement for wheels to be inside or outside of rear bumpers.

10) The technical officer can enforce seals on engines in any location that he requires with or without specification in supp regs.

11) Karts and or engines can be impounded by technical officer at any time for conformity checks.

12) All karts must be weighted after all races unless scale marshall deems it not required.

13) If your engine is not listed in this document you must contact Geelong Kart Club on 0447 607 796 and enquire as to eligibility for practise of the engine.

14) Cameras of any type are not allowed to be fitted in any way to a drivers helmet.  Cameras must be fitted to the kart in a manner which does not create any undue safety concern to any driver.  A tether must also be fitted to the camera if one is used.

15) Tyre tests to determine if any types of tyre treatments have been used by drivers can be conducted at any time by GKC Technical officer including during practise.

16) Fuel tests to determine use of any illegal substance that does not constitute part of the standard fuel purchased at the pump can be conducted by the GKC technical officer at any time including during practise.

*** Continual adjustments will be made at the discretion of Geelong Kart Club as required.   All final decisions on technical matters will be determined by the GKC Chief Technical Officer.  No appeals will be allowed. ***